GSM-R Certification Measurements

M150 line measurement report

Ferencváros - Soroksár – Kelebia OH

2026.03.11

Connecting line 154

For internal use only!!!!

# Table of contents

1	Table of contents	2

2	Version control	3

3	References	3

4	Introduction	3

5	Measurement path and measurement settings	3

6	Measurement data files	4

The measurements run are summarized in the table below.	4

7	Network configurations	5

7.1	NSS Network	5

7.2	BSS Network	5

7.3	Traffic load	5

8	Measuring wagon configurations	5

9	Track use conditions	7

10	Evaluation of measurements	7

10.1	Radio Coverage [RC]	7

11	Events table	9

12	Attachments	11

# Version control

| Version number | Release Date | Created by | Change |
| --- | --- | --- | --- |
| v01 | 2026.03.11 | Nagyszegi Béla | First version |

Board 1: Version Control Table

# References

The measurement report refers to the methods and requirements described in other documents. In the following, the report gives the number of the referenced document and its relevant chapter.

- BUBE_Coverage_and_QoS_Acceptance_Test_Plan_20260204_V1.0_Final

- Annex_9_Detailed_measurement_schedule_with_parameters _20260204_v1.0

- Subset_093_v4_0_0

- O_2475_v4_0_0

# Introduction

The purpose of this document is to present the line measurements and the evaluation of the measurements related to the certification of the GSMR network on the M150 line (on the OH section of the Soroksár-Kelebia). The measurements were carried out on 25.02.2026 based on the ATP Measurement instructions.

# Measurement path and measurement settings

The measurements are taken by the ATP, it was carried out on the following routes.

| Name | GSM-R Section Identifier | Maximum track speed | Distance (m) | Starting Point in Hectometers | End Point in Hectometers | Starting Point WGS LAT/LON | End Point WGS LAT/LON |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Kiskunhalas [Kh]– Kunfehértó [Kf] | M154 | 100km/h | 3980m | 55+10 | 1+00 | 46.391792, 19.467858 | 46.417344, 19.500797 |

Table 2: Completed Test Passes

The measurement settings used are shown in the following table.

| Drive test number | Drive test route | Elapsed time [min] Target speed [km/h] | Network mode | Load | Test terminal | Measurement script | Measured KPI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | Kh-Kf | 10 min 20 km/h | N | N | MOB7 | A41 | MATVR, HOSR, HOET, CLR |
|  |  |  |  |  | MOB8 | A42 | MATVR, HOSR, HOET, CLR |
|  |  |  |  |  | MOB9 | A21 | TD, HOSR, HOET, CLR |
|  |  |  |  |  | MOB10 | A22 | TD, HOSR, HOET, CLR |
|  |  |  |  |  | MOB11 | B1 | CST, CSER |
|  |  |  |  |  | MOB12 | B2 | HOSR, HOET, CLR, RQ |
|  |  |  |  |  | R-CW | S | RC |
| 24 | Kf-Kh | 10 min 20 km/h | N | N | MOB7 | A41 | MATVR, HOSR, HOET, CLR |
|  |  |  |  |  | MOB8 | A42 | MATVR, HOSR, HOET, CLR |
|  |  |  |  |  | MOB9 | A21 | TD, HOSR, HOET, CLR |
|  |  |  |  |  | MOB10 | A22 | TD, HOSR, HOET, CLR |
|  |  |  |  |  | MOB11 | B1 | CST, CSER |
|  |  |  |  |  | MOB12 | B2 | HOSR, HOET, CLR, RQ |
|  |  |  |  |  | R-CW | S | RC |
| 25 | Kh-Kf | 10 min 20 km/h | N | N | MOB7 | A1 | CED, CEER |
|  |  |  |  |  | MOB8 | A1 | CED, CEER |
|  |  |  |  |  | MOB9 | B2 | HOSR, HOET, CLR, RQ |
|  |  |  |  |  | MOB10 | B2 | HOSR, HOET, CLR, RQ |
|  |  |  |  |  | MOB11 | A22 | TD, HOSR, HOET, CLR |
|  |  |  |  |  | MOB12 | A42 | MATVR, HOSR, HOET, CLR |
|  |  |  |  |  | R-CW | S | RC |
| 26 | Kf-Kh | 10 min 20 km/h | N | N | MOB7 | A1 | CED, CEER |
|  |  |  |  |  | MOB8 | A1 | CED, CEER |
|  |  |  |  |  | MOB9 | B2 | HOSR, HOET, CLR, RQ |
|  |  |  |  |  | MOB10 | B2 | HOSR, HOET, CLR, RQ |
|  |  |  |  |  | MOB11 | A22 | TD, HOSR, HOET, CLR |
|  |  |  |  |  | MOB12 | A42 | MATVR, HOSR, HOET, CLR |
|  |  |  |  |  | R-CW | S | RC |

Board 3: Line measurement settings

The scripts run on each meter are described in the	BUBE_Coverage_and_QoS_Acceptance_Test_Plan _20260204_V1.0_Final[1] Acceptance Test

The definition, measurement method, requirements and method of evaluation of the measured KPIs can be described in the [1] Acceptance Test Protocol Measurement Description and Procedure v03

# Measurement data files

# The measurements run are summarized in the table below.

| Starting time | Starting point | End time | Ending point | test scenario | Measurement file |
| --- | --- | --- | --- | --- | --- |
| 16:26 | Kiskunhalas | 16:40 | Kiskunhalas - Kunfehértó H55 | sc25 | 20260311_162659 |
| 16:40 | Kiskunhalas - Kunfehértó H55 | 17:06 | Kiskunhalas | sc26 | 20260311_164026 |
| 17:09 | Kiskunhalas | 17:21 | Kiskunhalas - Kunfehértó H55 | sc23 | 20260311_170959 |
| 17:21 | Kiskunhalas - Kunfehértó H55 | 17:38 | Kiskunhalas | sc24 | 20260311_172107 |

Board 4: Completed Probes

# Network configurations

## NSS Network

The NSS network was checked on both measurement days before the measurements, and the logs certifying this are included in Appendix J.

## BSS Network

The BSS network was checked on all three measurement days before the measurements.

## Traffic load

During the measurement, at least 3TS load was applied to the Kontron/Huawei BTS TCH TRX operating on the Ferencváros – Kelebia line section, thus ensuring that all TCH TRX were switched on and broadcasting.

Command log files and OSS reports for traffic load are included in Appendix N.

# Measuring wagon configurations

The measurements are taken by the [1] Acceptance Test Protocol Measurement Description and Procedure final with the MW2 measuring wagon described in Chapter 4.1. The software/firmware versions of the measuring devices installed in the measuring wagon and participating in the measurement are summarized in the following table.

| Measuring device name | Type | Software/Firmware Version |
| --- | --- | --- |
| MOB7 | Triorail TRC-5RM | REV 03.040 |
| MOB8 | Triorail TRC-5RM | REV 03.040 |
| MOB9 | Triorail TRC-5RM | REV 03.040 |
| MOB10 | Triorail TRC-5RM | REV 03.040 |
| MOB11 | Triorail TRC-5RM | REV 03.040 |
| MOB12 | Triorail TRC-5RM | REV 03.040 |
| MOB14 | Triorail TRM-3AT | REV 03.040 |
| Netprobe |  | 3.1 |
| Netprobe Analytics |  | 3.28.30 |

Board 5: Software and firmware versions

The data of the radio devices and their corresponding SIMs in the measuring car are shown in the following table.

| Measuring device name | Info | IMSI | ICCID | IMSI |
| --- | --- | --- | --- | --- |
| MOB1 | MCA - MT2_1 (8W) | 216998800036310 | 89216991808010080151 | 8803631 |
| MOB2 | MCB - MT2_2 (8W) | 216998800036320 | 89216991808010080169 | 8803632 |
| MOB3 | MCC - MT2_3 (8W) | 216998800036330 | 89216991808010080177 | 8803633 |
| MOB4 | MCD - MT2_4 (8W) | 216998800036340 | 89216991808010080185 | 8803634 |
| MOB5 | MCE - MT2_5 (8W) | 216998800036350 | 89216991808010080193 | 8803635 |
| MOB6 | MCF - MT2_6 (8W) | 216998800036360 | 89216991808010080201 | 8803636 |
| MOB7 | MCG - TRC-5RM_1 (8W) | 216998800036370 | 89216991808010080219 | 8803637 |
| MOB8 | MCH - TRC-5RM_2 (8W) | 216998800036380 | 89216991808010080227 | 8803638 |
| MOB9 | MCI - TRC-5RM_3 (8W) | 216998800036390 | 89216991808010080235 | 8803639 |
| MOB10 | MCJ - TRC-5RM_4 (8W) | 216998800036400 | 89216991808010080243 | 8803640 |
| MOB11 | MCK - TRC-5RM_5 (8W) | 216998800036410 | 89216991808010080250 | 8803641 |
| MOB12 | MCL - TRC-5RM_6 (8W) | 216998800036420 | 89216991808010080268 | 8803642 |
| MOB13 | MA - TRM-3AT_1 (2W) | 216998800036430 | 89216991808010080276 | 8803643 |
| MOB14 | MB - TRM-3AT_2 (2W) | 216998800036440 | 89216991808010080284 | 8803644 |

Board 6: SIM card details

Before starting the measurement, the Triorail 5RM advanced receiver option was turned off on the Triorail 5RM mobile phones with the help of the Trio Trace 2 application by deactivating the disable interference resistance option from the "Forcing Functions" options. This step has been performed for each system reboot. The antenna connections of the mobile phones participating in the measurement are shown in the following table.

| Measuring device name | Antenna port (patch panel) | Antenna ID |
| --- | --- | --- |
| MOB7 | A1 | ANT1 |
| MOB8 | A2 | ANT2 |
| MOB9 | A3 | ANT3 |
| MOB10 | A4 | ANT4 |
| MOB11 | A5 | ANT9 |
| MOB12 | A6 | ANT5 |
| MOB14 | I4 | ANT10 |
| TSME6_1 | A7 | ANT6 |
| TSME6_2 | A8 | ANT7 |
| TSME6_3 | A9 | ANT11 |

Board 7: Antenna Connections

# Track use conditions

According to the regulations, the measurements must be carried out at the maximum speed allowed for the line, but in most cases this is not feasible due to traffic and technical reasons.

Maximum allowed speed during measurement : 20 km/h

# Evaluation of measurements

The definition, measurement method, requirements and method of evaluation of the measured KPIs can be described in the Acceptance Test Protocol Measurement Description and Procedure

The raw measurement results are included in Appendix A.

## Radio Coverage [RC]

The results were evaluated from 2 measurement passes over a distance of 300 km. The values measured with the scanner were evaluated using the built-in processing module of Netprobe Analytics. The measured data is processed in the following steps:

- Averaging according to Lee criterion

- GSM-R Channel Selection (955-973)

- Envelope calculation (selection of the strongest measured signal level)

- Calculation of 95% location probability for each 100m section

The processing is shown in the flowchart of Netprobe Analytics in the following diagram.

Figure 1: Scanner result processing flowchart

The evaluated sections are listed in Annex B. To calculate the time probability, we calculated the 95% probability based on all samples.

| KPI Name | KPIs | Threshold level | Measured value | Success/Sample | Review |
| --- | --- | --- | --- | --- | --- |
| Radio Coverage [RC] | 100m section ≥-95 dBm @95% | 100% | 100% | 12675/12675 | PASS |
| Radio Coverage [RC] | 95% Time Probability | ≥-95 dBm | -69.15 dBm | 12675 samples | PASS |

Board 9: Radio Coverage Measurement Results

The calculated Envelope values are shown in the graph below.

Figure 2: Scanner Envelop values as of 09/02/2026

# Events table

The table below shows the device events received during the measurements.

| Direction | MOB7 | MOB8 | MOB9 | MOB10 | MOB11 | MOB12 |
| --- | --- | --- | --- | --- | --- | --- |
| Connecting lines - H55 |  |  |  |  |  |  |
| Cell Reselection | 7 | 9 | 1 |  | 9 |  |
| Handover Attempt | 8 | 8 | 5 | 12 | 13 | 14 |
| Handover Complete | 8 | 8 | 5 | 12 | 13 | 14 |
| Voice Call Start |  |  | 2 | 2 | 38 | 2 |
| Voice Call Connect |  |  | 2 | 2 | 38 | 2 |
| Voice Call Finish |  |  | 2 | 2 | 38 | 2 |
| Data Call Start | 85 | 87 | 5 | 2 | 2 | 2 |
| Data Call Connect | 85 | 87 | 5 | 2 | 2 | 2 |
| Data Call Finish | 85 | 87 | 5 | 2 | 2 | 2 |

The table below shows the rx level statistics of devices. It shows that all requirements are passed.

| RxLevel | Avg | Max | Min |
| --- | --- | --- | --- |
| L1/L2 |  |  |  |
| Connecting lines - H55 |  |  |  |
| MOB7 | -58.82 | -30.00 | -83.00 |
| MOB8 | -58.76 | -29.00 | -84.00 |
| MOB9 | -56.66 | -27.00 | -86.00 |
| MOB10 | -59.50 | -28.00 | -84.00 |
| MOB11 | -58.90 | -27.00 | -84.00 |
| MOB12 | -58.48 | -25.00 | -85.00 |

The table below shows the overall rx quality distribution of devices. It shows that all requirements are passed, good quality probability is over 95%.

| RxQuality | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L1/L2 |  |  |  |  |  |  |  |
| Connecting lines - H55 |  |  |  |  |  |  |  |
| MOB7 | 97.88% | 0.40% | 0.42% | 0.62% | 0.29% | 0.33% | 0.07% |
| MOB8 | 98.30% | 0.46% | 0.49% | 0.28% | 0.26% | 0.21% | 0.00% |
| MOB9 | 99.30% | 0.31% | 0.18% | 0.15% | 0.03% | 0.02% | 0.00% |
| MOB10 | 97.80% | 0.63% | 0.52% | 0.52% | 0.26% | 0.26% | 0.01% |
| MOB11 | 97.89% | 0.58% | 0.52% | 0.35% | 0.23% | 0.38% | 0.06% |
| MOB12 | 95.56% | 1.10% | 1.05% | 0.99% | 0.77% | 0.44% | 0.08% |

The table below shows the required Rx Quality (range 0-3)  probability in space and time, evaluated over 1 km section blocks per MOBs. The quality degradation occurring in one section is normal due to the GSM technology (handover, cell selection), external interference section is not detected.

|  | Connecting lines - H55 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Distance in km | MOB7 | MOB8 | MOB9 | MOB10 | MOB11 | MOB12 |
| 1 | 99.76% | 99.32% | 100.00% | 99.85% | 99.53% | 99.61% |
| 2 | 100.00% | 99.74% | 100.00% | 99.19% | 99.51% | 98.60% |
| 3 | 100.00% | 99.69% | 100.00% | 97.49% | 98.30% | 98.45% |
| 4 | 97.24% | 98.12% | 100.00% | 98.20% | 97.68% | 97.72% |
| 5 | 99.19% | 99.84% | 99.85% | 99.94% | 99.93% | 98.70% |
| 6 | 100.00% | 99.78% | 100.00% | 99.74% | 100.00% | 100.00% |
| 7 | 98.22% | 99.54% | 100.00% | 99.37% | 100.00% | 99.38% |

The map below shows the handover attempts of devices. It shows that all requirements are passed. There was no failed handovers (ping-pong handover not occured)

The map below shows the voice call attempts of devices. It shows that all requirements are passed. There was no failed, dropped voice calls only in serbian section.

The map below shows the data call attempts of devices. It shows that all requirements are passed. There was no failed, dropped data calls only in serbian section.

# Attachments

The RAW data files for the measurements can be accessed at the following link:https://drive.google.com/drive/u/0/folders/1o9sZCTODhxjWZQQEgAOdyzFjD26alIgA